Technical Dossier

Technical Dossier DT-001

Zone 1 Complete Machine Datasheets

WANKOE limestone processing line, deterministic model output

| Dossier number | DT-001 |
| --- | --- |
| Status | ISSUED |
| Issue date | 2026-08-11 |
| Zones covered | 1.1 (crushing/primary screening) · 1.2 (reclaim/AgLime) · 1.3 (drying/grits/UltraFin) |
| Operating point | 250 t/h wet line feed · dry weather · mode 1A (KFS) / mode 2A (AgLime loop) · reclaim 100 t/h · FeedLime reclaim 30 t/h |
| Production target driving hours | KFS 85 000 t/y (FIRM) |
| Engine provenance | commit 679330f · wankoe_model (run_scenario / flowsheet / models M1-M8) · 2026-08-11 · 110 tests passed |
| Replay | python dossiers/DT-001/extract_dt001.py reproduces every figure (raw output archived as dt001_data.json) |
| Model reference | Expert book “Modèles mathématiques des machines” v2026-08-08 (docs/), first-principles derivations of every model used here |

Issued by NOEZYS LTD, AI Innovation Lab

2026-08-11

Our values: Authenticity, Impact, Creativity

# 1.  Validated input data (client-validated 2026-08-11)

Feed rate: 250 t/h wet at line inlet = 232.5 t/h dry at 7 % moisture (wet basis).

Feed PSD: measured belt-cut at the primary crusher outlet, 2026-08-08, average of tests 2 & 3: 45 % passing 19 mm, 52 % @ 40, 65 % @ 80, 76 % @ 120, 81 % @ 200. F80 = 180.6 mm; 19 % coarser than 200 mm.

Curve completed by hypotheses H-FEED-1 (fine tail < 19 mm from the reference-curve shape) and H-FEED-2 (top size 320 mm, 90 % @ 250 mm).

# 2.  Calculation hypotheses in force

| Hypothesis | Value | Status |
| --- | --- | --- |
| Bond work index Wi | 12.54 kWh/t | [ref.], Q2 closed 2026-08-11 (Fontaine, Belgian limestone) |
| Screen imperfection I (dry) | 0.15 | client arbitration Q1 (2026-08-10); expert clarification pending on 0.10 alternative |
| Screen imperfection I (rain) | 0.9 (cut collapses → mode 2B forced) | spec rule |
| Impact breakage A, b | 60 / 0.80 (A·b = 48) | [H], Q12 open; calcite drop-weight range A 62-69, b 1.3-3.0 |
| CR.5011 capacity | 90 t/h real (75-90), motor 132 kW | machine documents, Q8 closed 2026-08-11 |
| SP.36 cut | 65 µm (d50c = 0.65·d97) | Q6 closed 2026-08-11 (expert book ch.11) |
| CL.38 geometry | b = 0.14 m, Ne = 5, v_in = 15 m/s | design assumption (book ch.12), vendor drawing pending |
| ML.26 λ / S_att | 4.11 / 0.171 | [F], S_att 0.206 (book) vs 0.171 under expert arbitration |
| Mechanical efficiency η_m | 0.75 | project value |
| Dryer η_th / I_ev | 0.60 / 45 kg/m³/h | [H] |
| Φ(<65 µm) natural fines content | not measured, UltraFin NOT CERTIFIED | measurement pending (sieve/laser) |

# 3.  Line summary at the DT-001 operating point

| Product | Rate | P80 | Quality |
| --- | --- | --- | --- |
| KFS 20/35 | 51.35 t/h wet | 31.30 mm | COMPLIANT 30/55/15 (7.5 / 85.7 / 6.8 %) |
| AgLime 0/1.7 | 49.46 t/h wet | 1.065 mm | 98.8 % < 1.7 mm (no numeric spec exists) |
| FeedLime grits 2-4 | 7.40 t/h dry | 3.54 mm | 6.5 % > 4 mm (book's ≤ 5 % criterion NOT met at I = 0.15, expert clarification pending) |
| FeedLime fines 0/1.5 | 19.51 t/h dry | 1.16 mm | - |
| UltraFin < 65 µm | 0.99 t/h dry | 50 µm | NOT CERTIFIED until Φ measured |

Hours (computed from targets, hours follow production, never the reverse): zone 1.2: 3 713 h / 7 500 (49.5 %); zone 1.3: 6 753 h / 7 500 (90.0 %). Securing lever = open question Q7.

CAPACITY WARNING Zone 1.1 requires 2 069 h vs 2 000 h ceiling (103.5 %): the 85 kt firm KFS commitment does NOT fit the 1-shift/5-day regime at 250 t/h. Securing lever = open question Q7.

Mass balances: closed on dry solids in all three zones and on water in zone 1.3 (relative gaps < 10⁻⁶).

# 4.  Machine datasheets

## 4.1  CR.5009 - Toothed roll crusher (zone 1.1)

| Feed | 232.5 t/h dry (250 wet) · F80 180.6 mm |
| --- | --- |
| Settings | gap g = 40 mm (range 20-60) · x80 = g (client-validated) · n = 1.35 [ref.] |
| Models | M1 truncated Rosin-Rammler (1.7·x80, sub-x80 bypass) · M2 Bond |
| Product | 232.5 t/h dry · P80 27.96 mm · reduction 6.5:1 |
| Power | W = 0.455 kWh/t · P_net 105.8 kW · P_installed 141.0 kW (no vendor value to compare) |

ALERT Feed F80 181 mm > max nip 150 mm, saturation risk; 19 % of feed > 200 mm (design flag, upstream primary station to adjust).

Product curve (% passing): 2 mm 7.9 · 5 mm 23.4 · 10 mm 44.5 · 15 mm 58.7 · 20 mm 68.8 · 25 mm 76.2 · 31.5 mm 84.1 · 35 mm 87.6 · 40 mm 91.9 · 50 mm 96.2 · 63 mm 99.3 · 80 mm 100.

## 4.2  SR.5007 - Double-deck screen 35/20 mm (zone 1.1)

| Feed (converged, incl. recycle) | 261.97 t/h dry · P80 26.56 mm |
| --- | --- |
| Settings | a = 35 / 20 mm · I = 0.15 → s = 13.52 · d50c = a (k_d = 1 [H]) |
| Models | M3 Karra partition per deck · M4 VSMA area (Q_b = 14·a^0.6, f_p = 1.2, f₀ = 0.347, book flags Q_b heuristic “to validate”) |
| Products | +35: 29.47 t/h → CR.5011 · KFS 20-35: 47.76 dry / 51.35 wet · 0/20: 184.74 dry / 198.65 wet → stockpile |
| KFS envelope 30/55/15 | 7.48 / 85.73 / 6.78 %, COMPLIANT |
| Sizing | required area: top 6.80 m² · bottom 7.56 m² (limiting), installed area not provided (vendor gap) |

KFS curve (% passing): 15 mm 0.2 · 20 mm 7.5 · 25 mm 41.2 · 31.5 mm 81.1 · 35 mm 93.2 · 40 mm 99.2.  0/20 curve: 2 mm 11.2 · 5 mm 33.3 · 10 mm 63.9 · 15 mm 84.9 · 20 mm 98.0.

## 4.3  CR.5011 - Impact crusher HAZEMAG AP-S 1010 (zone 1.1)

| Machine data (Q8) | 100 t/h nominal · 75-90 t/h real on limestone · rotor 1000×1000 · max feed 400 mm · motor 132 kW · rotor-bound · mode 1B: line feed ≲ 130 t/h |
| --- | --- |
| Feed | 29.47 t/h dry (33 % of real capacity) · F80 54.97 mm |
| Settings | v = 45 m/s · CSS/x80 = 20 mm · A/b = 60/0.80 [H] (Q12) |
| Models | M5 JKMRC (E_cs = v²/7200 = 0.2813 kWh/t → t₁₀ = 12.09 % → n = 1.313) · M1 (trunc. 34 mm → 100 % < 35, vendor doc says ~95 %) · M2 Bond |
| Product | 29.47 t/h · P80 18.67 mm → recycle to SR.5007 |
| Power | W = 0.383 kWh/t · P_net 11.3 kW at loop equilibrium · 34.5 kW at 90 t/h capacity (spec's “~37 kW” reading confirmed) · motor utilization 35 % max |

## 4.4  SR.5105 - Reclaim screen 15/5 mm (zone 1.2)

| Feed | 93.0 t/h dry (100 wet reclaim) · P80 13.66 mm (zone 1.1's 0/20 curve) |
| --- | --- |
| Settings | a = 15 / 5 mm · I = 0.15 |
| Mode routing | 2A (current): mid 5-15 → FeedLime, rest → AgLime loop · 2B (rain): all → FeedLime · 2C: all → loop |
| Products | +15: 14.75 t/h · FeedLime 5-15: 47.00 dry / 50.54 wet · 0-5: 31.25 t/h |
| Sizing | required dry: 3.81 / 2.94 m²; rain case is sizing case: 5.08 / 3.92 m², installed area not provided |

## 4.5  SR.5115 - Closing screen 1.7 mm (zone 1.2)

| Feed | fresh loop 46.00 t/h · converged screen feed 83.74 t/h dry · circulating load 82 % (book floor ≈ 86 %, set by screen sharpness) |
| --- | --- |
| Settings | a = 1.7 mm (at Karra validity limit a > 1 mm, book ch.2) · I = 0.15 · rain: I 0.9 → cut impossible → mode 2B |
| Structural law (book ch.6) | closed loop ⇒ AgLime out = fresh loop in (46.00 = 46.00 verified); crusher setting only drives circulating load |
| Product | AgLime 0/1.7: 46.00 dry / 49.46 wet · P80 1.065 mm · 98.8 % < 1.7 mm |
| Sizing | Q_b = 19.2 t/h/m² → required 8.26 m² (largest screen of the plant), installed not provided |

## 4.6  CR.5107 - Impact crusher, AgLime loop (zone 1.2)

| Feed | 37.74 t/h dry · F80 17.17 mm |
| --- | --- |
| Settings | v = 40 m/s · CSS = 1.0 mm (= book recommendation CSS 1.0-1.2, v ≈ 40) · A/b = 60/0.80 [H] |
| Models | M5 (E_cs = 0.2222 → t₁₀ = 9.77 % → n = 1.400) · M1 · M2 |
| Product | 37.74 t/h · P80 0.95 mm · 97.5 % < 1.5 mm |
| Power | W = 3.12 kWh/t (heaviest specific duty of the line) · P_net 117.7 kW · P_installed 156.9 kW, vendor motor unknown (gap) |

## 4.7  DY.03 - Rotary dryer + burner BU.04 (zone 1.3)

| Feed | 30.0 t/h wet = 27.9 dry · 7 % → 0.5 % moisture |
| --- | --- |
| Models | M6 mass balance + energy balance incl. solid sensible heat (identical to book ch.8) |
| Output | 28.04 t/h wet · evaporated water 1.96 t/h |
| Power | thermal duty 2 085 kW · burner 3 475 kW (η_th = 0.60 [H]) |
| Sizing | required drum volume 43.6 m³ (I_ev = 45 [H]), installed burner/drum not provided (vendor gap) |

## 4.8  SN.21 - Classification screen 4/2/1.5 mm (zone 1.3)

| Feed | 64.20 t/h dry (27.9 dried + 36.3 recycle) · P80 9.40 mm · dry screening at 0.5 % moisture |
| --- | --- |
| Settings | a = 4 / 2 / 1.5 mm · I = 0.15 |
| Products | +4: 31.01 → ML.26 · grits 2-4: 7.40 t/h · sliver 1.5-2: 5.29 → ML.26 · fines 0-1.5: 20.50 → SP.36 |
| Grits quality | 15.0 % < 2 mm · 78.5 % in cut · 6.5 % > 4 mm, book's ≤ 5 % criterion NOT met at I = 0.15 (met at the book's sharper I ⇔ ours ≈ 0.10; expert clarification pending; no formal spec criterion wired) |
| Sizing | 3.57 / 4.20 / 3.97 m² (limiting: 2 mm deck), installed not provided |

## 4.9  ML.26 - Toothed roller mill, compressed bed, closed circuit (zone 1.3)

| Feed | 36.30 t/h dry (+4 & sliver) · F80 11.51 mm |
| --- | --- |
| Settings | gap 4 mm · λ = 4.11 [F] · S_att = 0.171 [F] (book: 0.206, under arbitration) · vendor product curve NOT provided (best upgrade, slot ready) |
| Models | M7 two-mechanism bed comminution (compression + attrition, progeny finer than parent, book ch.10 / Schönert) · M2 Bond |
| Product | 36.30 t/h · P80 2.59 mm → SN.21 · circulating load 1.30 (book ref ≈ 1.43 at sharper I) |
| Power | W = 1.30 kWh/t · P_net 47.1 kW · P_installed 62.8 kW (book ≈ 45 kW installed), grade C model |

## 4.10  SP.36 - Air classifier (zone 1.3)

| Feed | 20.50 t/h dry · P80 1.12 mm · Φ(<65 µm) modelled 6.44 %, unmeasured |
| --- | --- |
| Settings | cut = 65 µm (Q6: d50c = 0.65·d97, Stokes, book ch.11) · η_cl = 0.75 [H] |
| Products | UltraFin 0.99 t/h · P80 50 µm (5.3 kt/y at computed hours vs 4 kt target) · remainder 19.51 t/h stays FeedLime fines |
| Sizing | Q_air ≈ 1 980 m³/h |

STATUS NOT CERTIFIED until Φ measured (sieve/laser), highest-value pending measurement after the feed curve.

## 4.11  CL.38 - Cyclones (zone 1.3)

| Settings | D ≈ 0.7 m · b = 0.14 m (0.2D Stairmand, design assumption, vendor drawing pending) · Ne = 5 [H] · v_in = 15 m/s |
| --- | --- |
| Model | M8 Lapple d50 |
| Result | d50 = 4.23 µm, bulk of UltraFin captured |

ALERT < 4 µm tail leaves with cleaned air: polishing bag filter REQUIRED and absent from the flowsheet, design review item.

# 5.  Vendor data gaps blocking design closure

Installed screen areas (SR.5007, SR.5105, SR.5115, SN.21)

CR.5107 motor rating

DY.03 burner rating and drum volume

ML.26 vendor product curve

CL.38 cyclone drawing

Polishing bag filter (equipment missing from the flowsheet)

# 6.  Open questions affecting these figures

Q1-bis/Q3 imperfection convention & value (0.15 vs ≈ 0.10, drives grits quality and yield)

Q4 KFS envelope formalization

Q7 KFS securing lever (zone 1.1 hours 103.5 %)

Q12 A/b hardness pair

S_att 0.171 vs 0.206

Φ measurement

Site Bond test

Engine-run provenance: commit 679330f, functions run_scenario / run_required_hours / flowsheet.zone_1_1/1_2/1_3 / models.M1-M8, run date 2026-08-11, 110 tests passed. Every figure in this dossier is engine output (no assistant-computed values). Replay: python dossiers/DT-001/extract_dt001.py.